Kitchen 5V Addressable LEDs

Kitchen Cabinet Upper & Lower:

Total length of LED strips ordered 16.4 x 2 = 32.8 ft

Total length of cabinet = 34.8 ft

Materials:

- 16 AWG, 6 ft, 13A Power Cord

- 5V 60A Transformer Switching Driver Power Supply

- ESP8266 Wi-Fi Module

- BTF LED Strips 60/m IP30

- BTF LED corner connectors

- Aluminum Channel 12mm

- 18 AWG Silicone Stranded Wire (Multicolored)

- Helping Hands Soldering Tool

- Flux Paste

- Soldering Iron Holder w/Brass Coil Cleaner

Step 1 - Install Power cord to 5V60A power supply:

Step 2 - Cut the LED wires:

Step 3 - Strip all (3) LED Wires

Step 4 – ESP8266 Wi-Fi Board (3) wires

- Pair Red to Red – Twist together that’s going to the VIN pin on the module to the red voltage wire connected to the LED lights

- Pair Black to White – Twist together, black wire is coming from Wi-Fi module to the GND white wire on the LED lights

- Pair Green to Green – Data wires

Step 5 – Terminate wires to the 5V 60A power supply (Unplugged Power Supply).

Positive Terminal +V:

- Terminate the red wires in the 1st slot.

Negative Terminal -V:

- Terminate the black and white wires in the 4th slot.

Step 6 – Download Software from GitHub (macOS Version)

- Flasher Software - Flasher App (Load the flash drive first)

- WLED Software - WLED Software (Download the latest software version)

Step 7 – Run Software

- Run the flasher software

- Plug in the ESP8266 Wi-Fi Module to computer with a USB cable

- Run the 2nd software which is the firmware

Step 8 – Connect the ESP8266 to Wi-Fi

- Click on WLED-AP to connect

- Enter Default Password (lowercase): wled1234

- Click Next

- Automatically sends you to the WLED Homepage

- If you’re not transferred to the website, in the URL type in 4.3.2.1

Step 9 – WLED Home Page

- Click on WIFI SETTINGS

- Enter in the SSID (Network Name)

- Enter Network Password

- Create a name you can remember htttp://looseCannon.local (Example)

- Click Save & Connect

- Close out the website

Step 10: Pick up the ESP8266

- Hit the “RST” button on the left of the charging cable. This will save everything you have entered.

- Then, make sure the device is connected to the Wi-Fi

Step 11 – URL address and WLED Settings

- Type in your URL looseCannon.local (Example)

- Click Config > LED preferences

- Enter in Total LED count:

- Hit Save > Back

- Go to the color wheel and make sure the colors match up

- If colors do not match up go to Config > LED Preferences

- Scroll down to LED outputs:

- Play with the color order to get a color match you desire